Cakewalk StudioWare

for the

Roland DEP-3 Digital Effects Processor

Version 1.0d • April 1999

StudioWare Panel, Graphics, and Programming Copyright©1999 Li'l Chips Systems, All rights reserved.

Unauthorized duplication or distribution prohibited by law.

The DEP-3 StudioWare Panel Roland DEP-3.CakewalkStudioWare allows you to directly modify the Digital Equalizer and Digital Reverb Programmable parameter settings in real-time.  You can also record any changes made to these parameters in a Cakewalk track.

The Li'l Chips Systems web site also has a Cakewalk Instrument Definition File (.ins) for the DEP-3 providing easy Patch Selection (Program Change) by the Effect Name within Cakewalk.

For this file in addition to other DEP-3 resources and files, go to http://www.lilchips.com/cakewalk and http://www.lilchips.com/roland .

The roland.effects Newsgroup for the Roland DEP-3 can be found at news://news.lilchips.com .

For additional News Service information go to http://www.lilchips.com/newsserv .

Contents

StudioWare Panel Requirements

The DEP-3 StudioWare Panel

Real-Time Control

Auto-Track Control

Taking a Panel Snapshot

Additional DEP-3 MIDI Control

Li'l Chips Systems

Revision Guide

StudioWare Panel Requirements

To successfully use this StudioWare Panel you will require the following:

• A PC running Windows 95/98, a Pentium 200 with 64MB RAM or better is recommended

• A Video Display Adapter and Color Monitor, 800x600 in 256 colors or higher is recommended

• Cakewalk version 6.01 or newer

• A MIDI Interface

• A Roland DEP-3 Digital Effects Processor

The DEP-3 StudioWare Panel

The DEP-3 StudioWare Panel duplicates the Programmable areas of the Roland DEP-3 Digital Effects Processor.

The StudioWare Panel is designed to resemble the front panel layout of the DEP-3.  Once you are familiar with the basic operation of the digital effects processor, you should feel comfortable using this StudioWare Panel.

TRACK Knob

This widget allows you to set the Cakewalk Track that you wish to record all other widget movements to.

Tracks 1 through 64 of the 256 total Cakewalk tracks are supported, to make choosing a track easier with the mouse.

If you are using this panel for real-time control only, you must set this widget to the current track or a track with MIDI information otherwise the other widgets will be disabled.

The widget range is 1 to 64.

The default value of this widget is 1.

Right-clicking on this widget will let you choose the Port.

Set the Port to the MIDI Port that the DEP-3 is connected to.

Normally the Port number is set in the Cakewalk Track.

PROGRAMMABLE AREA Knobs

The set of eight knob widgets in the Programmable Area allow you to control the effect parameters of the DEP-3.

The Output knob widget sets the output level of the Effect.

The Digital Equalizer knob widgets adjust the Low, Mid and High frequency characteristics of the Effect. See the DEP-3 Owner’s Manual for information on adjusting the Digital Equalizer.

The Digital Reverb knob widgets adjust the Reverb characteristics, including Pre-Delay, Reverb Time, High Frequency Damp, and Reverb Select.  See the DEP-3 Owner’s Manual for information on adjusting the Digital Reverb.

Note:  The Input knob on the DEP-3 is not part of the Programmable area and therefore cannot be controlled from the StudioWare Panel.

DEVICE ID Knob

This widget allows you to set the StudioWare Panel's Device ID number to match the current Device ID of the DEP-3.  The Device ID is identical to the current DEP-3 MIDI Channel.

The widget range is 1 to 16, corresponding to MIDI Channel 1 to 16.

The default value of this widget is 1.

To determine the current DEP-3 MIDI Channel / Device ID, hold down the MANUAL/MIDI CH button on the DEP-3 unit for approximately three seconds, the red MIDI CH LED will illuminate and the green LED Numeric Display will show the current MIDI Channel.  See your DEP-3 Owner's Manual page 24.

ABOUT Button

The About button widget displays a popup with the StudioWare Panel copyright information.

OMNI Button

The Omni button widget allows you to toggle Omni Mode ON or OFF.

When Omni is OFF, the DEP-3 will respond to MIDI Patch Change commands only on its designated MIDI Channel.

When Omni is ON, the DEP-3 will respond to MIDI Patch Change commands on any MIDI Channel. Omni On mode is indicated by the small green LED lit beside the Device ID.

Omni Mode can also be set on the DEP-3 by holding the MANUAL/MIDI CH button for approximately three seconds, the red MIDI CH LED will illuminate and the green LED Numeric Display will show the current MIDI Channel.  Using the MEMORY NO. buttons you can select the MIDI Channel and OMNI Mode.

Turn to page 25 in your DEP-3 Owner’s Manual.

Notes:

* This StudioWare Panel modifies the current parameter settings in the DEP-3's Temporary area.  No changes are actually made to the DEP-3's internal presets.  To modify and save DEP-3 settings into the preset areas, see the DEP-3 Owner's Manual page 22.  To restore the DEP-3 Factory presets (only preset 1 through 20 can be restored to factory settings), see the DEP-3 Owner's Manual page 23.  Notes and other DEP-3 information can also be found on the Li'l Chips web site.

* The Program Change parameters of the DEP-3 have not been included on this Panel.  Use the Li'l Chips Systems DEP-3 Cakewalk INS file for Patch/Program Changes.  The Omni Mode can also be set on the DEP-3 unit itself.  See the DEP-3 Owner's Manual pages 24-25.

* This Panel has been tested on the Canadian/North-American version of the DEP-3 and is not guaranteed to work with other units from other countries.

This Panel sends the full 6-bit value (0 to 63) to the DEP-3 on all knobs with the exception of REVERB SELECT which sends the value range of 1 to 11 (0 to 10 internally), and the DEVICE ID knob range of 1 to 16 (0 to 15 internally) as stated above.

According to the Roland DEP-3 Owner's Manual MIDI Implementation (Pg.31):

"The actual values obtained on the DEP-3 differ from the values sent with MIDI ( #0 - #63 ).  From the Roland distributor in your country, you can attain the table that shows how the MIDI values correspond to the actual values on the DEP-3."

Real-Time Control

1. Connect your DEP-3 unit to your PC's MIDI Out.  Turn on the DEP-3 power switch.

2. Adjust the Panel TRACK knob to the current Cakewalk track or a track that has MIDI events.

This will enable the Panel knobs if they are currently grayed out.

3. Adjust the Panel DEVICE ID knob to the MIDI channel that the DEP-3 is currently set to.

To determine the current DEP-3 MIDI Channel / Device ID, hold down the MANUAL/MIDI CH button on the DEP-3 unit for approximately three seconds, the red MIDI CH LED will illuminate and the green LED Numeric Display will show the current MIDI Channel.  See your DEP-3 Owner's Manual page 24.

4. Adjust the PROGRAMMABLE knobs on the StudioWare panel to change the current DEP-3 settings in real-time.

Notes:

* Each Panel control movement sends a total of 17 bytes of System Exclusive data (from Sysex F0h to F7h inclusive) to the MIDI Port.

Auto-Track Control

1. Connect your DEP-3 unit to your PC's MIDI Out.  Turn on the DEP-3 power switch.

2. Open an existing Cakewalk .wrk file for a song you wish to add DEP-3 events to.

3. Adjust the Panel TRACK knob to the current Cakewalk track that you wish to record events to (the track with the current selection rectangle).  This will enable the Panel knobs if they are currently grayed out.

4. Adjust the Panel DEVICE ID knob to the MIDI channel that the DEP-3 is currently set to.

To determine the current DEP-3 MIDI Channel / Device ID, hold down the MANUAL/MIDI CH button on the DEP-3 unit for approximately three seconds, the red MIDI CH LED will illuminate and the green LED Numeric Display will show the current MIDI Channel.  See your DEP-3 Owner's Manual page 24.

5. Press the Record button on the DEP-3 StudioWare Panel.

6. Press the Cakewalk Play button to start playing the current .wrk file.

7. Adjust the PROGRAMMABLE knobs on the StudioWare panel to change the current DEP-3 settings.

The StudioWare knob changes will be recorded to the current track.

Notes:

* Search the Cakewalk on-line Help for the keywords StudioWare or Recording Widgets Movements for additional information.

* Each Panel control movement records a total of 17 bytes of System Exclusive data (from Sysex F0h to F7h inclusive) to the track.

Taking a Panel Snapshot

Taking a Snapshot inserts an event capturing the position of each knob into the current track, at the current time.

When you have arranged the Panel knobs the way you want, you can take a snapshot of their positions by pressing the camera button on the Panel.  This allows you to save a "preset" or "starting" set of parameter values for the DEP-3 in a Cakewalk track.

Notes:

* A Panel snapshot records System Exclusive data to the track.

Additional DEP-3 MIDI Control

The Roland DEP-3 accepts the following MIDI messages:

Program Change	Cm pp

where m = MIDI Channel 0 to F hex (corresponding to 1 to 16)

and pp = Program Number 00 to 62 hex (corresponding to 1 to 99)

Omni Mode		Bm 7C 00	= Omni Off

Bm 7D 00	= Omni On

where m = MIDI Channel 0 to F hex (corresponding to 1 to 16)

System Exclusive	See the DEP-3 Owner’s Manual.

See the DEP-3 Owner's Manual MIDI Implementation pages 30 and 31.

Li'l Chips Systems

Email:			lilchips@lilchips.com

lilchips@compuserve.com

Main Web Site:		http://www.lilchips.com

CompuServe Page:	http://ourworld.compuserve.com/homepages/lilchips

FTP Site:		ftp://ftp.lilchips.com

Roland Newsgroups:	news://news.lilchips.com

DISCLAIMER

This "Product" (including any software and documentation and files) is provided without warranty of any kind.  You, the end user, assume all responsibility for your use of this Product.

This “Product” may be freely distributed only in its entirety, all files and copyright markings must remain together as a single package and intact.  All other rights are reserved.

This “Product” is for non-commercial use only, it is not to be included with any other products or released on any media such as CD-ROM without the prior written consent of Li'l Chips Systems.

You may not sell this “Product” for profit, or disassemble or duplicate the “Product”.

This product may include information held under copyright of other companies or persons.

Revision Guide

1.0d	28 April 1999

• Added the OMNI Mode button and LED indicator.  Updated documentation.

• Minor changes to the Panel graphics (StudioWare logo in bottom left).

1.0c	23 April 1999

• New About Popup graphic.

1.0c	14 April 1999

• Added About Button and About Popup.

• New Graphics.

• Optimized Panel for smaller screen size (from 700x108 down to 662x98).

This is a decrease to 86% of the previous version.

• Optimized Panel for smaller file size (from 227kb down to 91kb).

This is a decrease to 40% of the previous version

• Updated documentation.

• Included Word 97 and RTF versions of the documentation.

1.0b	21 March 1999

• Changed “MIDI CH / DEV ID” Panel text to “DEVICE ID” to avoid confusion.

• Updated documentation.

1.0a	March 1999

• Initial Version.

Document Rev 1.0e April 28 1999 DRG