Cakewalk StudioWare

for the

Roland JV and XP-series 64 Voice Synthesizers

Roland JVXP Performance CC Mixer Panel

Version 1.0a • July 1999

StudioWare Panel, Graphics, and Programming Copyright©1999 Li'l Chips Systems, All rights reserved.

Unauthorized duplication or distribution prohibited by law.

This Cakewalk StudioWare Panel is designed for the Roland JV and XP-series 64 Voice Synthesizer Modules and Workstations.

The JV/XP StudioWare Panel Roland JVXP Perform CC Mixer.CakewalkStudioWare allows you to directly control many of the JV/XP Synthesizer's Performance Parameters using Control Change Messages in real-time.  You can also record any changes made to these parameters in a Cakewalk track.

The Li'l Chips Systems web site has numerous Cakewalk Files, information and resources for the JV and XP-series Synthesizers.  Go to http://www.lilchips.com/cakewalk and http://www.lilchips.com/roland .

The roland.synth.superjvxp.series Newsgroup for the Roland JV and XP-series Synthesizers can be found at news://news.lilchips.com .

For additional News Service information go to http://www.lilchips.com/newsserv .

Contents

StudioWare Panel Requirements

Conventions Used in This Document

The JV/XP StudioWare Panel

Real-Time Control

Auto-Track Control

Taking a Panel Snapshot

Additional JV/XP MIDI Control

Li'l Chips Systems

Revision Guide

StudioWare Panel Requirements

To successfully use this StudioWare Panel you will require the following:

• A PC running Windows 95/98, a Pentium 200 with 64MB RAM or better is recommended

• A Video Display Adapter and Color Monitor, 800x600 in 256 colors or higher is recommended

• Cakewalk version 6.01 or newer

• A MIDI Interface

• A Roland JV or XP-series 64 Voice Synthesizer Module or Workstation

Conventions Used in This Document

• JV/XP refers to the Roland 6A Family of 64 Voice Synthesizers and Workstations.  At the time of this document this includes the JV-1010, JV-1080, and JV-2080 Modules and the XP-30, XP-50, XP-60, and XP-80 Workstations.

• Where any Synthesizer display page or value is referenced in this document, only those models with the large graphic LCD display interface will be similar.  At the time of this document this includes the JV-2080, XP-60, and XP-80.

• Words enclosed in square brackets [ ] indicate Synthesizer Panel buttons.

Example:  [PERFORM] indicates the PERFORM button.

• The order in which buttons are to be pressed will often be indicated by words enclosed in square brackets [ ], together with + and .

Example:  [SHIFT] + [PERFORM]  [F4] (Part)

means that you are to hold down the SHIFT button and press the PERFORM button, and then press the F4 (Part) button.

• Display Page navigation in this document refers specifically to the JV-2080.  The actual Synthesizer Panel buttons used to get to a display page may vary on the other Synthesizer models.

Example:  [PERFORM]  [F5] (Effects) page

will take you to the Performance Effects page on the JV-2080 Synthesizer.

• A StudioWare Panel widget refers to any of the StudioWare Panel controls including buttons, knobs, sliders, etc.  See your Cakewalk User's Guide for additional information on StudioWare.

• A StudioWare Panel popup refers to a secondary dialog or popup window on a StudioWare Panel.  See your Cakewalk User's Guide for additional information on StudioWare.

The JV/XP StudioWare Panel

The JV/XP Performance CC Mixer StudioWare Panel allows remote parameter control of many the Performance Part Parameters of the Roland JV and XP-series 64 Voice Synthesizers using Control Change messages.

The advantage of using Control Change messages over System Exclusive messages is their smaller size, normally only a few bytes of data versus a dozen or more for each Sysex message.

A known limitation of MIDI is the amount of data that can be transferred at any one time.

The numerous Performance parameters of this Panel can also be accessed directly on the JV/XP Synthesizer’s front panel and by choosing the [PERFORM]  [F3] (Part), [PERFORM]  [F4] (MIDI), and [PERFORM]  [F5] (Effects) pages.  The Patch Tone Level, TVF, and TVA parameters can be accessed for each Part’s Patch in Performance Mode by choosing [PERFORM] + [PATCH].

JV-2080 owner's can see pages 16 (Parts of a Performance), 17 (Selecting Performances), 53 (Modifying the Chorus Settings), 54 (Modifying the Reverb Settings), 63-69 (Creating a Performance), 69 (Creating Patches for Each Part While still in Performance Mode), 83 (TVF Param page), and 85 (TVA Param page) in their Owner's Manual.

See the Owner’s Manual MIDI Implementation Chart for the entire list of Control Change messages supported by the JV/XP Synthesizer.

The main StudioWare Panel is designed to resemble the buttons and look of a JV/XP Synthesizer design.  Once you are familiar with the basic operation of your Synthesizer, you should feel comfortable using this StudioWare Panel.

The Panel is divided into 16 Channel Strips, one for each Performance Part.

The rightmost Strip is used as a main Panel control area.

Panel Control Strip

TRACK Knob

This widget allows you to set the Cakewalk Track that you wish to record all other widget movements to.

All Control Change events for all Performance Parts are recorded to this one track.

All of the Cakewalk tracks (1 through 256) are supported.

If you are using this Panel for real-time control only you must set this widget to the current track or a track with MIDI information, otherwise the other widgets will be disabled.

The default value of this widget is 1.

The widget range is 1 to 256.

DEV ID Knob

This widget allows you to set the StudioWare Panel's Device ID number to match the current Device ID of the JV/XP Synthesizer.  The Device ID Range is 17 through 32.

The default value of this widget is 17.

The widget range is 17 to 32.

Choose the [SYSTEM]  [F3] (MIDI) page on the Synthesizer.

See your JV/XP Owner's Manual on how to determine your Synthesizer's Device ID.

JV-2080 owner's can turn to page 136 in their Owner's Manual.

Note: The RX button widgets are the only Panel widgets that rely on the Device ID setting.

ABOUT Button

Choosing the About button widget displays the StudioWare Panel copyright, version, and contact information.

Channel Strip Area

Each of the Channel Strips directly correspond to a JV/XP Performance Part.

The Channel Strips are preset to MIDI Channels 1 through 16 as denoted by the number across the top.

Channel Strip Number Indicator

The Channel Number Indicator denotes the MIDI Channel and Performance Part of each strip.

RX Button

Rx Switch (Receive Switch).

Specifies whether each Part will receive MIDI messages (ON), or not (OFF).

In other words, this specifies whether each Part will be used (ON) or not (OFF).

This widget uses System Exclusive messages to communicate with the JV/XP.

When the indicator of this button is lit, it is ON.

Choose the [PERFORM]  [F4] (MIDI) page on the Synthesizer.

JV-2080 owner's can turn to page 67 in their Owner's Manual.

REVERB Send Level Knob

The Reverb Send Level knob widget determines the amount of Reverb Effect on the Part.

This is Control Change # 91 (General Purpose Effect 1).

The Reverb Send Level range is from 0 (none) to 127 (full).

The default value of this widget is 0.

Choose the [PERFORM]  [F5] (Effects)  [F6] (Palette) page on the Synthesizer to view the entire palette of Reverb Send Level values.  Choose the [F1] button on the Synthesizer to toggle between the two pages for Performance Parts 1-8 and 9-16.

CHORUS Send Level Knob

The Chorus Send Level knob widget determines the amount of Chorus Effect on the Part.

This is Control Change # 93 (General Purpose Effect 3).

The Chorus Send Level range is from 0 (none) to 127 (full).

The default value of this widget is 0.

Choose the [PERFORM]  [F5] (Effects)  [F6] (Palette) page on the Synthesizer to view the entire palette of Chorus Send Level values.  Choose the [F1] button on the Synthesizer to toggle between the two pages for Performance Parts 1-8 and 9-16.

Tone Level 1 to 4 Sliders [1] [2] [3] [4]

This set of four slider widgets allows you to set the volume level of each of the four Tones that comprise a Patch.

This is Control Changes # 80 through 83 (General Purpose Controller 5 through 8).

Each Tone Level range is from 0 (none) to 127 (full).

The default value of this widget set is 100.

Choose the [PERFORM] + [PATCH]  [F4] (TVA) page on the Synthesizer to view the Tone Level values.  Choose the [F6] (Palette) button on the Synthesizer to view all four Tones.

JV-2080 owner's can turn to page 85 in their Owner's Manual.

TVF / TVA Sliders [C] [R] [A] [D]

This set of four slider widgets allows you to adjust the TVF Cutoff [C], TVF Resonance [R], TVA Attack Level [A], and TVA Decay Level [D] on a Patch.

This is Control Changes # 71 through 74 (Sound Controller 2 through 5).

Each slider range is from 0 (none) to 127 (full).

The default value of this widget set is 100.

Choose the [PERFORM] + [PATCH]  [F3] (TVF) page on the Synthesizer to view the TVF Cutoff and Resonance values.

JV-2080 owner's can turn to pages 83-84 in their Owner's Manual.

Choose the [PERFORM] + [PATCH]  [F4] (TVA)  [F2] (TVA Env) page on the Synthesizer to view the TVA Attack and Decay values.

JV-2080 owner's can turn to page 86 in their Owner's Manual.

PAN Knob

This knob widget allows you to set the Panning Location from Left to Right for a Performance Part.

This is Control Change # 10.

The widget range is 0 to 127.

The default value of this widget is 64 (Center).

Choose the [PERFORM]  [F3] (Part) page on the Synthesizer.

JV-2080 owner's can turn to pages 66-67 in their Owner's Manual.

EXPRESSION Knob

This knob widget allows you to set the Expression Level for a Performance Part.

This is Control Change # 11.

The widget range is 0 to 127.

The default value of this widget is 100.

Note: You can use the combination of the Expression and Volume widgets to define a “maximum” level for a track and a “variable” level, by setting one of them to the loudest desired volume while using the range of the other to adjust the current track level within that range.

This helps to perform sound level changes without ever having to memorize or to prevent losing your “maximum” value setting.

Note: Expression will not affect Part Volume unless the VOL&EXP setting on the Synthesizer is enabled.

Choose the [SYSTEM]  [F4] (Control)  [F4] (Control) page on the Synthesizer.

For additional information on selecting which MIDI messages will control Volume, see your JV/XP Owner’s Manual.

JV-2080 owner's can turn to page 140 in their Owner's Manual.

VOLUME Slider

This slider widget allows you to set the Volume Level for a Performance Part.

This is Control Change # 7.

Choose the [PERFORM]  [F3] (Part) page on the Synthesizer.

JV-2080 owner's can turn to pages 66-67 in their Owner's Manual.

Notes:

• This StudioWare Panel modifies the current parameter settings in the JV/XP's Temporary Area.  No changes are actually made to the JV/XP's internal presets.  To modify and save JV/XP settings into the Synthesizer’s User or Memory Card areas, see your JV/XP Owner's Manual.  The Memory Card Area is for applicable JV-series only.

• This StudioWare Panel uses Control Change messages to modify the current parameter settings in the JV/XP.  Unlike sending System Exclusive messages, you may not see the specific parameter value change in real-time on the JV/XP LCD display.

• You can use the combination of the Expression and Volume widgets to define a “maximum” level for a track and a “variable” level, by setting one of them to the loudest desired volume while using the range of the other to adjust the current track level within that range.

This helps to perform sound level changes without ever having to memorize or to prevent losing your “maximum” value setting.

• This StudioWare Panel was designed and tested on a Pentium PC using Cakewalk Pro Audio version 6.01.

• This StudioWare Panel has been tested on the Canadian/North-American version of the JV-2080 and is not guaranteed to work with other JV/XP units from other countries.

Real-Time Control

1. Connect your JV/XP unit to your PC's MIDI port.  Turn on the JV/XP power switch.

2. Adjust the Panel TRACK knob to the current Cakewalk track or a track that has MIDI events.

This will enable the Panel widgets if they are currently disabled (grayed out).  This Track must have its Port value set to the MIDI Port that the JV/XP Synthesizer is located on.

3. Adjust the Panel DEV ID knob to the Device ID that the JV/XP is currently set to.

See your JV/XP Owner's Manual for this information.

4. Adjust the desired widgets on the StudioWare Panel to change the current JV/XP settings in real-time.

Notes:

* Each Panel widget movement sends a Control Change message to the Panel’s Track with the exception of the RX widget which uses System Exclusive messages.

* Multiple widgets can be “grouped” together to perform such operations as simultaneous level changes across multiple parts.  To group widgets, hold down the Ctrl key while selecting each desired widget with the mouse.  Release the Ctrl key after all of the desired widgets have been selected.  Moving any of the grouped widgets will now move the entire group.

To ungroup, hold the Ctrl key and select a widget in the group, then release the Ctrl key.  Perform this same operation for all other widgets in the group.

Auto-Track Control

1. Connect your JV/XP unit to your PC's MIDI port.  Turn on the JV/XP power switch.

2. Open an existing Cakewalk .wrk file for a song you wish to add JV/XP events to.

3. Adjust the Panel TRACK knob to the current Cakewalk track that you wish to record events to (the track with the current selection rectangle).  This will enable the Panel controls if they are currently disabled (grayed out). This Track must have its Port value set to the MIDI Port that the JV/XP Synthesizer is located on.

4. Adjust the Panel DEV ID knob to the Device ID that the JV/XP is currently set to.

See your JV/XP Owner's Manual for this information.

5. Press the Record button on the JV/XP StudioWare Panel.

6. Press the Cakewalk Play button to start playing the current .wrk file.

7. Adjust the desired widgets on the StudioWare Panel to change the current JV/XP settings.

The StudioWare widget changes will be recorded to the track defined by the TRACK knob.

Notes:

* Search the Cakewalk on-line Help for the keywords StudioWare or Recording Widgets Movements for additional information.

* All Channel Strip widget events are recorded into one track, as designated by the Track knob widget.

* Each Panel widget movement sends a Control Change message to the Panel’s Track with the exception of the RX widget which uses System Exclusive messages.

* Multiple widgets can be “grouped” together to perform such operations as simultaneous level changes across multiple parts.  To group widgets, hold down the Ctrl key while selecting each desired widget with the mouse.  Release the Ctrl key after all of the desired widgets have been selected.  Moving any of the grouped widgets will now move the entire group.

To ungroup, hold the Ctrl key and select a widget in the group, then release the Ctrl key.  Perform this same operation for all other widgets in the group.

* During playback of your sequence, if you set the Update button on the StudioWare Panel to “in”, the widgets will automatically update their positions to correspond to the events in each track.

Taking a Panel Snapshot

Taking a Snapshot inserts an event capturing the position of each widget into the current track, at the current time.

When you have arranged the Panel widgets the way you want, you can take a snapshot of their positions by pressing the camera button on the Panel.  This allows you to save a "preset" or "starting" set of parameter values for the JV/XP in a Cakewalk track.

Notes:

* This Panel contains a large number of widgets.  This will result in a large amount of snapshot data that will take a short time to send to the Synthesizer every time you press play or record in Cakewalk.

Additional JV/XP MIDI Control

The Roland JV and XP-series Synthesizers accept a large variety of MIDI System Exclusive messages.

See the JV/XP Owner's Manual MIDI Implementation, or get additional JV/XP StudioWare Panels from the Li'l Chips Systems web site.

Li'l Chips Systems

Email:			lilchips@lilchips.com

lilchips@compuserve.com

Main Web Site:		http://www.lilchips.com

CompuServe Page:	http://ourworld.compuserve.com/homepages/lilchips

FTP Site:		ftp://ftp.lilchips.com

Roland Newsgroups:	news://news.lilchips.com

DISCLAIMER

This "Product" (including any software and documentation and files) is provided without warranty of any kind.  You, the end user, assume all responsibility for your use of this Product.

This “Product” may be freely distributed only in its entirety, all files and copyright markings must remain together as a single package and intact.  All other rights are reserved.

This “Product” is for non-commercial use only, it is not to be included with any other products or released on any media such as CD-ROM without the prior written consent of Li'l Chips Systems.

You may not sell this “Product” for profit, or disassemble or duplicate the “Product”.

This product may include information held under copyright of other companies or persons.

Revision Guide

1.0a	20 July 1999

• Fixed minor bug with Volume Sliders 7 and 8 being linked.

1.0a	1 May 1999

• Initial Version

Known Bugs and Limitations:

The following are StudioWare limitations/peculiarities and not problems with the design of this Panel.  These bugs will not affect the general operation of this Panel.

• Most of the Panel widget values cannot be easily retrieved from the Synthesizer due to each parameter being located in individual Part Address blocks within the Synthesizer.

This is a limitation of StudioWare and not this Panel.

Document Rev 1.0a July 20 1999 DRG