Cakewalk StudioWare

for the

Roland JV and XP-series 64 Voice Synthesizers

Roland JVXP Performance Chorus Reverb II Panel

Version 1.0b • May 1999

StudioWare Panel, Graphics, and Programming Copyright©1999 Li'l Chips Systems, All rights reserved.

Unauthorized duplication or distribution prohibited by law.

This Cakewalk StudioWare Panel is designed for the Roland JV and XP-series 64 Voice Synthesizer Modules and Workstations.

The JV/XP StudioWare Panel Roland JVXP Perform Chorus Reverb II.CakewalkStudioWare allows you to directly control the JV/XP Synthesizer's Global Chorus and Reverb Parameters in addition to each of the Performance Part's Chorus and Reverb Send Levels in real-time.  You can also record any changes made to these parameters in a Cakewalk track.

The Li'l Chips Systems web site has numerous Cakewalk Files, information and resources for the JV and XP-series Synthesizers.  Go to http://www.lilchips.com/cakewalk and http://www.lilchips.com/roland .

The roland.synth.superjvxp.series Newsgroup for the Roland JV and XP-series Synthesizers can be found at news://news.lilchips.com .

For additional News Service information go to http://www.lilchips.com/newsserv .

Contents

StudioWare Panel Requirements

Conventions Used in This Document

The JV/XP StudioWare Panel

Real-Time Control

Auto-Track Control

Taking a Panel Snapshot

Additional JV/XP MIDI Control

Notes, Tips, and Tricks

Li'l Chips Systems

Revision Guide

StudioWare Panel Requirements

To successfully use this StudioWare Panel you will require the following:

• A PC running Windows 95/98, a Pentium 200 with 64MB RAM or better is recommended

• A Video Display Adapter and Color Monitor, 800x600 in 256 colors or higher is recommended

• Cakewalk version 6.01 or newer

• A MIDI Interface

• A Roland JV or XP-series 64 Voice Synthesizer Module or Workstation

Conventions Used in This Document

• JV/XP refers to the Roland 6A Family of 64 Voice Synthesizers and Workstations.  At the time of this document this includes the JV-1010, JV-1080, and JV-2080 Modules and the XP-30, XP-50, XP-60, and XP-80 Workstations.

• Where any Synthesizer display page or value is referenced in this document, only those models with the large graphic LCD display interface will be similar.  At the time of this document this includes the JV-2080, XP-60, and XP-80.

• Words enclosed in square brackets [ ] indicate Synthesizer Panel buttons.

Example:  [PERFORM] indicates the PERFORM button.

• The order in which buttons are to be pressed will often be indicated by words enclosed in square brackets [ ], together with + and .

Example:  [SHIFT] + [PERFORM]  [F4] (Part)

means that you are to hold down the SHIFT button and press the PERFORM button, and then press the F4 (Part) button.

• Display Page navigation in this document refers specifically to the JV-2080.  The actual Synthesizer Panel buttons used to get to a display page may vary on the other Synthesizer models.

Example:  [PERFORM]  [F5] (Effects) page

will take you to the Performance Effects page on the JV-2080 Synthesizer.

• A StudioWare Panel widget refers to any of the StudioWare Panel controls including buttons, knobs, sliders, etc.  See your Cakewalk User's Guide for additional information on StudioWare.

• A StudioWare Panel popup refers to a secondary dialog or popup window on a StudioWare Panel.  See your Cakewalk User's Guide for additional information on StudioWare.

The JV/XP StudioWare Panel

The JV/XP Performance Chorus Reverb II StudioWare Panel allows remote parameter control of the Global Chorus and Reverb of the Roland JV and XP-series 64 Voice Synthesizers.

The Global Chorus and Reverb can also be accessed on the JV/XP Synthesizer by choosing the [PERFORM]  [F5] (Effects) page.

JV-2080 owner's can see pages 25 (Turning Effects On/Off), 28-31 (Performance), 53 (Modifying the Chorus Settings), and 54 (Modifying the Reverb Settings) in their Owner's Manual.

The main StudioWare Panel is designed to resemble the buttons and look of a JV/XP Synthesizer design.  Once you are familiar with the basic operation of your Synthesizer, you should feel comfortable using this StudioWare Panel.

PORT Knob

This widget allows you to set the Cakewalk MIDI Port that the Synthesizer is connected to.

Ports 1 through 16 are supported.  Be sure to set this to the appropriate Port otherwise certain Panel functions will not work.

The default value of this widget is 1.

TRACK Knob

This widget allows you to set the Cakewalk Track that you wish to record all other widget movements to.

Tracks 1 through 64 of the 256 total Cakewalk tracks are supported, to make choosing a track easier with the mouse.

If you are using this Panel for real-time control only, you must set this widget to the current track or a track with MIDI information, otherwise the other widgets will be disabled.

The default value of this widget is 1.

DEV ID Knob

This widget allows you to set the StudioWare Panel's Device ID number to match the current Device ID of the JV/XP Synthesizer.  The Device ID Range is 17 to 32.

The default value of this widget is 17.

Choose the [SYSTEM]  [F3] (MIDI) page on the Synthesizer.

See your JV/XP Owner's Manual on how to determine your Synthesizer's Device ID.

JV-2080 owner's can turn to page 136 in their Owner's Manual.

EFFECTS ON/OFF Area (EFX CHORUS REVERB)

The EFFECTS ON/OFF Area duplicates the identical section on the JV/XP Synthesizer.

These three button widgets allow you to switch each of the Synthesizer Effects sections ON or OFF.

This is convenient when adjusting the parameters for one of the effects, in that you can turn the other two off so that they do not interfere with the precise effect sound that you are attempting to achieve.

See the JV/XP Synthesizer Owner's Manual for information on each Effect switch.

JV-2080 owners can turn to page 25 in the Owner's Manual.

ABOUT Button

Choosing the About button widget displays the StudioWare Panel copyright, version, and contact information.

CHORUS Parameter Area

The CHORUS Parameters area allows you to define the Chorus Effect.

LEVEL: Specify the volume of the sound routed through Chorus.

RATE: Specify the modulation frequency of the Chorus sound.

DEPTH: Specify the modulation depth of the Chorus sound.

PRE-DELAY: Specify the time delay from the original sound until the Chorus sound is heard.

FEEDBACK: Specify the amount of the sound which passed through Chorus that is to be returned to the input of the Chorus.

OUTPUT: Specify how the sound from the Chorus will be output (Mix, Reverb, Mix+Reverb).

The REQUEST button widget will retrieve the current Chorus Parameters from the JV/XP Synthesizer.

Choose the [PERFORM]  [F5] (Effects)  [F4] (Chorus) page on the Synthesizer.

See the JV/XP Synthesizer Owner's Manual for in-depth information on each parameter.

JV-2080 owners can turn to page 53 in the Owner's Manual.

CHORUS Level Area

The Chorus Level area allows you to specify each individual Performance Part's Chorus Send Level.

The Chorus Send Level range is from 0 (none) to 127 (full).

The R (Request) button widget beneath each of the level sliders allows you to retrieve the current Chorus Send Level value for the slider.

The “tick” marks located beside each slider correspond directly to MIDI Level values incrementing by 16s (0, 16, 32, 48, 64, 80, 96, 112, 127).  The dotted mark just above 96 is the MIDI Level 100.

Choose the [PERFORM]  [F5] (Effects)  [F6] (Palette) page on the Synthesizer to view the entire palette of Chorus Send Level values.  Choose the [F1] button on the Synthesizer to toggle between the two pages for Performance Parts 1-8 and 9-16.

See the JV/XP Synthesizer Owner's Manual for in-depth information on each parameter.

REVERB Parameter Area

The REVERB Parameters area allows you to define the Reverb Effect.

TYPE: Specify the type of Reverb or Delay.  Two Rooms, two Stages, two Halls, and two types of Delay are available.

LEVEL: Specify the volume of the Reverb or Delay sound.

TIME: Adjusts the length of Reverberation or Delay time.

HF DAMP: Specify the frequency at which the high frequency portion of the Reverb or Delay will be cut.  A value of 0Hz on the StudioWare Panel Display Area is equivalent to Bypass.

DELAY FEEDBACK: (Delay only) Specify the amount of Delay sound that is returned to the Delay input.

The REQUEST button widget will retrieve the current Reverb Parameters from the JV/XP Synthesizer.

Choose the [PERFORM]  [F5] (Effects)  [F5] (Reverb) page on the Synthesizer.

See the JV/XP Synthesizer Owner's Manual for in-depth information on each parameter.

JV-2080 owners can turn to page 54 in the Owner's Manual.

REVERB Level Area

The Reverb Level area allows you to specify each individual Performance Part's Reverb Send Level.

The Reverb Send Level range is from 0 (none) to 127 (full).

The R (Request) button widget beneath each of the level sliders allows you to retrieve the current Reverb Send Level value for the slider.

The “tick” marks located beside each slider correspond directly to MIDI Level values incrementing by 16s (0, 16, 32, 48, 64, 80, 96, 112, 127).  The dotted mark just above 96 is the MIDI Level 100.

Choose the [PERFORM]  [F5] (Effects)  [F6] (Palette) page on the Synthesizer to view the entire palette of Reverb Send Level values.  Choose the [F1] button on the Synthesizer to toggle between the two pages for Performance Parts 1-8 and 9-16.

See the JV/XP Synthesizer Owner's Manual for in-depth information on each parameter.

Notes:

• This StudioWare Panel modifies the current parameter settings in the JV/XP's Temporary Area.  No changes are actually made to the JV/XP's internal presets.  To modify and save JV/XP settings into the Synthesizer’s User or Memory Card areas, see your JV/XP Owner's Manual.  The Memory Card Area is for applicable JV-series only.

• This StudioWare Panel was designed and tested on a Pentium PC using Cakewalk Pro Audio version 6.01.

• This StudioWare Panel has been tested on the Canadian/North-American version of the JV-2080 and is not guaranteed to work with other JV/XP units from other countries.

Real-Time Control

1. Connect your JV/XP unit to your PC's MIDI port.  Turn on the JV/XP power switch.

2. Adjust the Panel PORT knob to the Cakewalk MIDI port that the Synthesizer is connected to.

This will enable all of the Panel features.

3. Adjust the Panel TRACK knob to the current Cakewalk track or a track that has MIDI events.

This will enable the Panel widgets if they are currently disabled (grayed out).

4. Adjust the Panel DEV ID knob to the Device ID that the JV/XP is currently set to.

See your JV/XP Owner's Manual for this information.

5. Select the desired buttons on the StudioWare Panel to change the current JV/XP settings in real-time.

Notes:

* Each Panel widget movement sends a Sysex message to the Panel’s Track and Port.

* Multiple widgets can be “grouped” together to perform such operations as simultaneous level changes across multiple parts.  To group widgets, hold down the Ctrl key while selecting each desired widget with the mouse.  Release the Ctrl key after all of the desired widgets have been selected.  Moving any of the grouped widgets will now move the entire group.

To ungroup, hold the Ctrl key and select a widget in the group, then release the Ctrl key.  Perform this same operation for all other widgets in the group.

* Choosing a Panel’s Request button will retrieve the values for that Panel Area from the Synthesizer.  Not all Panel areas support the Request feature.

Auto-Track Control

1. Connect your JV/XP unit to your PC's MIDI port.  Turn on the JV/XP power switch.

2. Open an existing Cakewalk .wrk file for a song you wish to add JV/XP events to.

3. Adjust the Panel PORT knob to the Cakewalk MIDI port that the Synthesizer is connected to.

This will enable all of the Panel features.

4. Adjust the Panel TRACK knob to the current Cakewalk track that you wish to record events to (the track with the current selection rectangle).  This will enable the Panel controls if they are currently disabled (grayed out).

5. Adjust the Panel DEV ID knob to the Device ID that the JV/XP is currently set to.

See your JV/XP Owner's Manual for this information.

6. Press the Record button on the JV/XP StudioWare Panel.

7. Press the Cakewalk Play button to start playing the current .wrk file.

8. Select the desired buttons on the StudioWare Panel to change the current JV/XP settings.

The StudioWare widget changes will be recorded to the track defined by the TRACK knob.

Notes:

* Search the Cakewalk on-line Help for the keywords StudioWare or Recording Widgets Movements for additional information.

* Each Panel widget movement sends a Sysex message to the Panel’s Track and Port.

* Multiple widgets can be “grouped” together to perform such operations as simultaneous level changes across multiple parts.  To group widgets, hold down the Ctrl key while selecting each desired widget with the mouse.  Release the Ctrl key after all of the desired widgets have been selected.  Moving any of the grouped widgets will now move the entire group.

To ungroup, hold the Ctrl key and select a widget in the group, then release the Ctrl key.  Perform this same operation for all other widgets in the group.

* During playback of your sequence, if you set the Update button on the StudioWare Panel to “in”, the widgets will automatically update their positions to correspond to the events in each track.

* Choosing a Panel’s Request button will retrieve the values for that Panel Area from the Synthesizer. Not all Panel areas support the Request feature.

Taking a Panel Snapshot

Taking a Snapshot inserts an event capturing the position of each widget into the current track, at the current time.

When you have arranged the Panel widgets the way you want, you can take a snapshot of their positions by pressing the camera button on the Panel.  This allows you to save a "preset" or "starting" set of parameter values for the JV/XP in a Cakewalk track.

Notes:

* This Panel contains a large number of widgets which use Sysex commands.  This will result in a large amount of snapshot data that will take a short time to send to the Synthesizer every time you press play or record in Cakewalk.

Additional JV/XP MIDI Control

The Roland JV and XP-series Synthesizers accept a large variety of MIDI System Exclusive messages.

See the JV/XP Owner's Manual MIDI Implementation, or get additional JV/XP StudioWare Panels from the Li'l Chips Systems web site.

Notes, Tips, and Tricks

Synthesizer Editing Real-time Feedback

• Setting the JV/XP’s Tx Edit Data Switch to ON will reflect any changes you make directly on the Synthesizer to the StudioWare Panel in real-time.

This feature lets both the StudioWare Panel and the Synthesizer maintain identical parameter values at all times, irregardless of which unit you make the modifications on.

Choose the [SYSTEM]  [F3] (MIDI) page on the Synthesizer to view the Overall Transmit/Receive Switches.

See the JV/XP Synthesizer Owner's Manual for in-depth information on each parameter.

JV-2080 owners can turn to page 136 in the Owner's Manual.

Retrieving the current Parameter Values from the Synthesizer

* Choosing one of the Panel Area’s Request buttons will retrieve the values for that Panel Area from the Synthesizer.

This feature lets the StudioWare Panel “initialize” its widgets to the current Synthesizer values.

Not all Panel areas support the Request feature.

Li'l Chips Systems

Email:			lilchips@lilchips.com

lilchips@compuserve.com

Main Web Site:		http://www.lilchips.com

CompuServe Page:	http://ourworld.compuserve.com/homepages/lilchips

FTP Site:		ftp://ftp.lilchips.com

Roland Newsgroups:	news://news.lilchips.com

DISCLAIMER

This "Product" (including any software and documentation and files) is provided without warranty of any kind.  You, the end user, assume all responsibility for your use of this Product.

This “Product” may be freely distributed only in its entirety, all files and copyright markings must remain together as a single package and intact.  All other rights are reserved.

This “Product” is for non-commercial use only, it is not to be included with any other products or released on any media such as CD-ROM without the prior written consent of Li'l Chips Systems.

You may not sell this “Product” for profit, or disassemble or duplicate the “Product”.

This product may include information held under copyright of other companies or persons.

Revision Guide

1.0b	1 May 1999

• Fixed the Device ID number range.

1.0a	27 April 1999

• Initial Version

Known Bugs and Limitations:

The following are StudioWare limitations/peculiarities and not problems with the design of this Panel.  These bugs will not affect the general operation of this Panel.

• The Chorus and Reverb Send Level slider widget values must be retrieved from the Synthesizer one at a time. All 16 slider values cannot be retrieved at one time with one button click. Therefore, a Request button widget is located under each Level slider.

This is a limitation of StudioWare and not this Panel.

Document Rev 1.0b May 1 1999 DRG