Cakewalk StudioWare

for the

Roland S-10 Digital Sampling Keyboard

Roland S-10 Performance Panel

Version 1.0a • April 1999

StudioWare Panel, Graphics, and Programming Copyright©1999 Li'l Chips Systems, All rights reserved.

Unauthorized duplication or distribution prohibited by law.

The S-10 StudioWare Panel Roland S-10 Performance.CakewalkStudioWare allows you to directly modify the Performance parameter settings in real-time.  You can also record any changes made to these parameters in a Cakewalk track.

The Li'l Chips Systems web site has a Cakewalk Instrument Definition File (.ins) for the S-10 providing easy Patch Selection (Program Change) by Sample Structure Name within Cakewalk.

The Li'l Chips Systems web site also has the entire Roland L-Series Sound Library available for free download, in addition to the entire Li’l Chips Systems Sample Libraries.

For these files in addition to other S-10 resources, go to http://www.lilchips.com/cakewalk and http://www.lilchips.com/roland .

The roland.sampler.s.series Newsgroup for the Roland S-10 can be found at news://news.lilchips.com .

For additional News Service information go to http://www.lilchips.com/newsserv .

Contents

StudioWare Panel Requirements

The S-10 StudioWare Panel

Real-Time Control

Auto-Track Control

Taking a Panel Snapshot

Additional S-10 MIDI Control

Li'l Chips Systems

Revision Guide

StudioWare Panel Requirements

To successfully use this StudioWare Panel you will require the following:

• A PC running Windows 95/98, a Pentium 200 with 64MB RAM or better is recommended

• A Video Display Adapter and Color Monitor, 800x600 in 256 colors or higher is recommended

• Cakewalk version 6.01 or newer

• A MIDI Interface

• A Roland S-10 Digital Sampling Keyboard

The S-10 StudioWare Panel

The S-10 StudioWare Panel allows access to the Performance parameters of the Roland S-10 Digital Sampling Keyboard.

The S-10 features various functions for controlling performance, such as pitch bender, vibrato, pedal hold and auto arpeggio.

To edit the Performance parameters on the S-10, use the following procedure:

1. Push the [PERFORMANCE] button

2. By using the [FORWARD] and [BACKWARD] button, select the performance parameter you wish to edit in the LCD display.

3. Rotate the Alpha-Dial to change the value of the parameter.

4. Repeat steps 2 and 3 to edit the other performance parameters as required.

5. Push the [ENTER] button when finished.

The StudioWare Panel allows access to all of the Sampling Structure Performance parameters with the exception of Trigger Play.  The Performance functions include:

Vibrato

Bender

Arpeggio

Velocity Mix

Velocity Switch

Detune

Delay

Trigger Play

TRACK Knob

This widget allows you to set the Cakewalk Track that you wish to record all other widget movements to.

Tracks 1 through 64 of the 256 total Cakewalk tracks are supported, to make choosing a track easier with the mouse.

If you are using this panel for real-time control only, you must set this widget to the current track or a track with MIDI information otherwise the other widgets will be disabled.  This Cakewalk track must have the Port value set to the MIDI port that the S-10 is connected to.

The widget range is 1 to 64.

The default value of this widget is 1.

DEVICE ID Knob

This widget allows you to set the StudioWare Panel's Device ID number to match the current Device ID of the S-10.  The Device ID is identical to the current S-10 MIDI Channel.

The widget range is 1 to 16, corresponding to MIDI Channel 1 to 16.

The default value of this widget is 1.

To determine the current S-10 MIDI Channel / Device ID, perform the following:

1. Push the [MIDI] button.

2. Use the [FORWARD] and [BACKWARD] buttons to select the MIDI Channel function in the LCD display.

3. Rotate the Alpha-Dial to the MIDI Channel you wish to use.

See your S-10 Owner's Manual page 61.

ABOUT Button

The About button widget displays a popup with the StudioWare Panel copyright information.

VIBRATO Area

RATE:  Sets the rate of the vibrato from 0 to 127.

M-VIB DPTH:  Sets the depth of the manual vibrato from 0 to 127.

D-VIB DPTH:  Sets the depth of the delay vibrato from 0 to 127.

D-VIB DLAY:  Sets the time needed for the delayed vibrato to come on after a key is played from 0 to 127.

If the vibrato in the Wave Parameter (S-10 Owner’s Manual page 43) is set to OFF, the sound will not take on vibrato at all.

See page 18 in the S-10 Owner’s Manual.

ARPEGGIO Area

Press the Arpeggio button widget or the corresponding Arpeggio button on theS-10 to enable the arpeggio function.

SYNC:  Sets the arpeggio sync to the internal rate or an external device.

RATE:  Sets the rate of the arpeggio from 0 to 127.

MODE:  Sets the shape of the arpeggio, Up, Down, Up/Down and Random.

RANGE:  Sets the number of octaves to be covered in the arpeggio, either 1, 2 or 3.

REPEAT:  Sets the number of times the notes/chord should be arpeggiated from 1 to 16.

DECAY:  Sets the amount of decay in the arpeggio from 1 to 10.

See page 19 in the S-10 Owner’s Manual.

BENDER Area

MODE:  Sets the Pitch Bender function to Continuous (smooth) or Chromatic (semi-tones).

See page 19 in the S-10 Owner’s Manual.

DETUNE Area

To enable Detune on the S-10, choose a Structure button, press the [F1] button, then the same Structure button.

MODE:  Sets the detune mode, FIX or VELO.  VELO increases the detune with harder playing.

RANGE:  Sets the pitch difference between the two detuned sounds from 0 to 127.

ABEND DEST:  Determines whether one or both voices in the detune are affected by the Auto Bend function.

BEND DEST: Determines whether one or both voices in the detune are affected by the Pitch Bend function.

See page 23 in the S-10 Owner’s Manual.

DELAY Area

To enable Delay on the S-10, choose a Structure button, press the [F2] button, then the same Structure button.

TIME:  Sets the amount of time between the direct and delayed sounds from 0 to 127.

LEVEL:  Sets the level of the delayed sound from 0 to 127.

KEY OFFSET:  Sets the pitch of the delayed sound in semi-tones from –12 to +12.

See page 24 in the S-10 Owner’s Manual.

VELOCITY Area

Velocity Mix and Velocity Switch are both used when the keyboard is in Dual Tone mode.

To enable Dual Tone on the S-10, press two Structure buttons at the same time.

To enable Dual Tone with Velocity Mix on the S-10, choose a Structure button, press the [F1] button, then choose another Structure button

To enable Dual Tone with Velocity Switch on the S-10, choose a Structure button, press the [F2] button, then choose another Structure button

V-MX THRSH:  Sets the keyboard playing threshold for velocity mixing from 0 to 127.

V-SW THRSH: Sets the keyboard playing threshold for velocity switching from 0 to 127.

See page 25 in the S-10 Owner’s Manual.

Real-Time Control

1. Connect your S-10 to your PC's MIDI connectors.  Turn on the S-10 power switch.

2. Set the desired Cakewalk track’s Port to the MIDI port that the S-10 is connected to.

3. Adjust the Panel TRACK knob to the above Cakewalk track.

This will enable the Panel knobs if they are currently grayed out.

4. Adjust the Panel DEVICE ID knob to the MIDI channel that the S-10 is currently set to.

To determine the current S-10 MIDI Channel / Device ID, perform the following:

a. Push the [MIDI] button.

b. Use the [FORWARD] and [BACKWARD] buttons to select the MIDI Channel function in the LCD display.

c. Rotate the Alpha-Dial to the MIDI Channel you wish to use.

See your S-10 Owner's Manual page 61.

5. Adjust the knob widgets on the StudioWare panel to change the current S-10 Performance parameters in real-time.

Notes:

* Each Panel widget movement sends System Exclusive data to the Track.

* Certain Panel features, Detune for example, requires that you set the appropriate mode on the S-10 before the widgets will have any audible effect.

Auto-Track Control

1. Connect your S-10 to your PC's MIDI connectors.  Turn on the S-10 power switch.

2. Open an existing Cakewalk .wrk file for a sequence you wish to add S-10 Performance events to.

3. Set the desired Cakewalk track’s Port to the MIDI port that the S-10 is connected to.

4. Adjust the Panel TRACK knob to the above Cakewalk track.

This will enable the Panel knobs if they are currently grayed out.

5. Adjust the Panel DEVICE ID knob to the MIDI channel that the S-10 is currently set to.

To determine the current S-10 MIDI Channel / Device ID, perform the following:

a. Push the [MIDI] button.

b. Use the [FORWARD] and [BACKWARD] buttons to select the MIDI Channel function in the LCD display.

c. Rotate the Alpha-Dial to the MIDI Channel you wish to use.

See your S-10 Owner's Manual page 61.

6. Press the Record button on the S-10 StudioWare Panel.

7. Press the Cakewalk Play button to start playing the current .wrk file.

8. Adjust the Performance knobs on the StudioWare panel to change the current S-10 Performance parameters.

The StudioWare knob changes will be recorded to the current track.

Notes:

* Search the Cakewalk on-line Help for the keywords StudioWare or Recording Widgets Movements for additional information.

* Each Panel widget movement sends System Exclusive data to the Track.

* Certain Panel features, Detune for example, requires that you set the appropriate mode on the S-10 before the widgets will have any audible effect.

Taking a Panel Snapshot

Taking a Snapshot inserts an event capturing the position of each knob into the current track, at the current time.

When you have arranged the Panel knobs the way you want, you can take a snapshot of their positions by pressing the camera button on the Panel.  This allows you to save a "preset" or "starting" set of parameter values for the S-10 in a Cakewalk track.

Additional S-10 MIDI Control

The Roland S-10 also accepts numerous additional MIDI messages:

This includes Notes, Controllers, Control Changes, and Sample Dumps.

See the S-10 Owner's Manual MIDI Implementation page 66.

Li'l Chips Systems

Email:			lilchips@lilchips.com

lilchips@compuserve.com

Main Web Site:		http://www.lilchips.com

CompuServe Page:	http://ourworld.compuserve.com/homepages/lilchips

FTP Site:		ftp://ftp.lilchips.com

Roland Newsgroups:	news://news.lilchips.com

DISCLAIMER

This "Product" (including any software and documentation and files) is provided without warranty of any kind.  You, the end user, assume all responsibility for your use of this Product.

This “Product” may be freely distributed only in its entirety, all files and copyright markings must remain together as a single package and intact.  All other rights are reserved.

This “Product” is for non-commercial use only, it is not to be included with any other products or released on any media such as CD-ROM without the prior written consent of Li'l Chips Systems.

You may not sell this “Product” for profit, or disassemble or duplicate the “Product”.

This product may include information held under copyright of other companies or persons.

Revision Guide

1.0a	23 April 1999

• Initial Version.

Document Rev 1.0a April 23 1999 DRG