Cakewalk StudioWare

for the

Roland SCC-1 GS Sound Card

Roland SCC-1 Part Rx Panel

Version 1.0c • April 1999

StudioWare Panel, Graphics, and Programming Copyright©1999 Li'l Chips Systems, All rights reserved.

Unauthorized duplication or distribution prohibited by law.

The SCC-1 StudioWare Panel Roland SCC-1 Part Rx.CakewalkStudioWare allows you to directly modify the MIDI Channel Receive Switch for each of the 16 Parts, the Master Volume, and whether Part 11 in addition to Part 10 is used for Rhythm.  A GS Reset button is also included.  All of these parameter changes are made directly to the SCC-1’s MIDI Port.

The Li'l Chips Systems web site has additional Roland SCC-1 and Cakewalk resources and files.  Go to http://www.lilchips.com/cakewalk and http://www.lilchips.com/roland .

The roland.soundcanvas Newsgroup for the Roland SCC-1 can be found at news://news.lilchips.com .

For additional News Service information go to http://www.lilchips.com/newsserv .

Contents

StudioWare Panel Requirements

The SCC-1 StudioWare Panel

Real-Time Control

Auto-Track Control

Taking a Panel Snapshot

Additional SCC-1 MIDI Control

Li'l Chips Systems

Revision Guide

StudioWare Panel Requirements

To successfully use this StudioWare Panel you will require the following:

• A PC running Windows 95/98, a Pentium 200 with 64MB RAM or better is recommended

• A Video Display Adapter and Color Monitor, 800x600 in 256 colors or higher is recommended

• Cakewalk version 6.01 or newer

• A MIDI Interface

• A Roland SCC-1 GS Sound Card*

* This StudioWare Panel may work with other GS-compatible sound devices.

The SCC-1 StudioWare Panel

The SCC-1 StudioWare Panel allows you to control sections of the SCC-1 that cannot be easily modified directly due to the absence of any front panel or controls on the sound card.

PART MIDI RX Buttons

This set of 16 button widgets allows you to toggle the MIDI Receive Switch for each of the 16 Parts.

Since the SCC-1 also has a built-in MPU-401 compatible MIDI Processing Unit, you may wish to disable MIDI Receive on some of the channels in order to use those channels for another external MIDI device such as a synthesizer.

Each button widget has a red LED indicator.

The LED indicator is lit when the Part’s MIDI Receive Switch is off (the Part will not receive MIDI messages).

PORT Knob

This knob widget allows you to specify the current Port number that the SCC-1 is mapped to in Cakewalk.

Normally this should be set to the Port Number that the Track View shows is mapped to the SCC-1’s MPU-401 MIDI In/Out.  If the Port value is incorrect, the Panel widgets will have no effect.

The widget range is 1 to 16.

The default value of this widget is 1.

MASTER VOLUME Knob

This knob widget allows you to set the overall volume of the SCC-1.

The widget range is 0 to 127.

The default value of this widget is 100.

CH11 = RHYTHM Button

This button widget allows you to set Part 11 to be a Rhythm Part, same as Part 10.

This lets you use two Percussion sets at the same time to create more complex rhythm sequences.

ABOUT Button

The About button widget displays a popup with the StudioWare Panel copyright information.

RESET Button

The Reset button widget sends a GS Reset command to the SCC-1.

Notes:

* This StudioWare Panel does not support Auto-Track features.  None of the widget values can be recorded to a Track.

Real-Time Control

1. The SCC-1 is an internal PC card, so operation is automatic when you turn on your computer.

2. Adjust the Panel PORT knob to the current Cakewalk Port that the SCC-1 is mapped to.

3. Adjust the Panel’s PART MIDI RX, CH11=RHYTHM, and MASTER VOLUME widgets as desired to change the current SCC-1 settings in real-time.

Notes:

* Each Panel control movement sends System Exclusive data to the MIDI Port.

Auto-Track Control

This StudioWare Panel does not support Auto-Track features.  None of the widget values can be recorded to a Track.

Taking a Panel Snapshot

This StudioWare Panel does not support Auto-Track features.  None of the widget values can be recorded to a Track.

Additional SCC-1 MIDI Control

The Roland SCC-1 accepts a wide range of MIDI messages, from Notes, to Controller Numbers, to System Exclusive.  See your SCC-1 Owner’s Manual for additional information.

Li'l Chips Systems

Email:			lilchips@lilchips.com

lilchips@compuserve.com

Main Web Site:		http://www.lilchips.com

CompuServe Page:	http://ourworld.compuserve.com/homepages/lilchips

FTP Site:		ftp://ftp.lilchips.com

Roland Newsgroups:	news://news.lilchips.com

DISCLAIMER

This "Product" (including any software and documentation and files) is provided without warranty of any kind.  You, the end user, assume all responsibility for your use of this Product.

This “Product” may be freely distributed only in its entirety, all files and copyright markings must remain together as a single package and intact.  All other rights are reserved.

This “Product” is for non-commercial use only, it is not to be included with any other products or released on any media such as CD-ROM without the prior written consent of Li'l Chips Systems.

You may not sell this “Product” for profit, or disassemble or duplicate the “Product”.

This product may include information held under copyright of other companies or persons.

Revision Guide

1.0c	23 April 1999

• Fixed the Port Number initialization bug.  Would initialize to 0 at loading.

1.0b	13 April 1999

• Completely redesigned the entire Panel.  Added graphics and more features.

• Included complete documentation.

1.0a	30 September 1997

• Initial Version.

Document Rev 1.0b April 23 1999 DRG